Ethernet Converter 100Mbps

www.takemind.com

manual

products

Product Manual V1.1

EC1001

100Base-T1 Ethernet Converter Introduction

The EC1001 Ethernet Converter seamlessly converts 100BASE-T1 automotive Ethernet to 100BASE-TX Fast Ethernet via an RJ45 interface. It supports bidirectional transmission in full-duplex mode, enabling reliable communication between standard computers and 100BASE-T1-based vehicle controllers.

Key Features

- MOLEX Mini50 automotive Ethernet interface

- 4.5V to 50V ultra-wide voltage input, withstanding up to -40V to +60V; compatible with 12V/24V systems

- Compliant with ISO 16750-2 and ISO 7637-2 testing standards for 12V/24V vehicles, suitable for in-vehicle use

- Supports multiple power supply interfaces: USB, Banana plug, and DC 5.0

- Link LED for real-time connection status; Rainbow LED for real-time communication quality monitoring

- DIP switches for effortless configuration

- Zero data modification for lossless, transparent transmission

Product Information

| Parameter | Description |
| --- | --- |
| Input Voltage： | 4.5~50VDC |
| Withstand Voltage Range: | -40~60VDC |
| Power Consumption： | <2W |
| Dimensions: | 69x104x30mm |
| Net Weight: | 175g |
| Operating Temperature: | -40℃~+80℃ |
| Protection Rating： | IP20 |

Package Contents

- 1 × 100Base-T1 Ethernet Converter

- 1 × 1 m USB power cable

- 1 × 30 cm banana-plug power cable

- 1 × Mini50 connector to quick connector*

* Custom adapter cables are available upon request.

Standard Use Case

The DIP switch configuration of the EC1001 depends on the device under test.If the device under test is a master, the EC1001 must be configured as a slave.Refer to the DIP Switch section for details.

DIP Switch

| DIP Switch | State | Description |
| --- | --- | --- |
| 1 | ON (up) | 100Base-T1 port set to Slave mode |
|  | OFF (down) | 100Base-T1 port set to Master mode |
| 2 | Reserved |  |

Note: In a 100Base-T1 link, one device must be configured as the master and the other as the slave. The 100Base-TX interface uses auto-negotiation.

LED Information

| Functional Domain | Indicator | State | Color | Description |
| --- | --- | --- | --- | --- |
| 100Base-T1 | ON | Steady on | Red | Power connected |
|  |  | Off | — | Power not connected |
|  | LN | Steady on | Red | 100Base-T1 link established |
|  |  | Blinking | Red | 100Base-T1 communication |
|  |  | Off | — | 100Base-T1 link not established |
|  | SQI | Steady on | Multicolor | Signal quality;Lower value indicates poorer signal (Red, Orange, Yellow, Green, Cyan, Blue, Purple: 1–7) |
|  |  | Off | — | No signal |
|  |  | Blinking | Red | Functional error |
| 100Base-TX | RJ45 Left | Steady on | Green | 100Base-TX link established |
|  |  | Off | — | 100Base-TX link not established |
|  |  | Blinking | Green | 100Base-TX communication |
|  | RJ45 Right | — | — | Reserved |

Application Scenarios

Scenario 1: Laboratory ECU Communication Debugging

This scenario demonstrates a typical connection between a vehicle controller and a host computer in a laboratory setting. The EC series device acts as a communication interface node, establishing a stable data exchange link between the controller and the PC via cables. Engineers can monitor, debug, and verify functions on the computer, ensuring the reliability and consistency of the communication process.

Scenario 2: In-Vehicle Environmental Debugging

This scenario showcases the application in an actual vehicle environment. The EC series interface device connects to the vehicle's ECU through an Automotive Ethernet port, enabling high-speed communication with external test systems. Used directly on the vehicle, it handles data access and transmission, supporting engineers in performing network debugging, data acquisition, and system validation under real driving conditions. This configuration is ideal for R&D road tests, control strategy verification, and real-time vehicle network diagnostics.

Scenario 3: Industrial Production Environment

In an industrial production control cabinet, the EC series Automotive Ethernet converter serves as a communication bridge for ECU testing. The device provides stable and reliable data conversion between Automotive Ethernet and Standard Ethernet. This configuration is widely utilized in ECU End-of-Line (EOL) testing processes.

Warranty Information

Please use this product under the conditions described in this manual.The warranty period is limited to one year. Do not disassemble the enclosure; otherwise, the warranty will be void.

Revision History

| Version | Section | Description | Date |
| --- | --- | --- | --- |
| 1.0 | All | Initial release | 2022/09/01 |
| 1.1 | Application Scenarios | Added application scenario description | 2025/10/21 |

Users can download the latest documentation for the EC1001 100Base-T1 Ethernet Converter at:

http://www.takemind.com/en/ec100x-en/

Contact Us

Shanghai Dazei Intelligent Technology Co., Ltd

Building 5, No. 288 Yulv Road, Jiading District, Shanghai, China

• +86(0)21-5997-0838

• support@takemind.com

• www.takemind.com